Part Number:	PT944-33190-02 (Gas)

PT944-33191-02 (Hybrid)

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Trunk Sill |
| 2 | 1 | Hardware Kit |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 11 | 15cm Wire Tie |
| 2 | 1 | Foam Tape 90mm x 30mm |
| 3 | 1 | Foam Tape 90mm x 60mm |
| 4 | 1 | Clip |
| 5 | 2 | Foam Tape 45mm x 30mm |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Conflicts

None

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| Safety Glasses |  |
| Safety Gloves | Optional |
| Vehicle Protection | Blanket |
| Special Tools | Notes |
| Panel Removal Tool | To remove Passenger Lower Dash Panel |
| Installation Tools | Notes |
| 10mm Socket / Ratchet or 10mm Wrench | For Battery |
| Screwdriver | Flathead |
| Diagonal cutting pliers | For Zip Tie cutting |
| Special Chemicals | Notes |
| Cleaner | VDC Approved Cleaner |

General Applicability

MY19 and newer ES/ESh vehicles

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 | Illuminated Trunk Sill |
| 2 | Cargo Net |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory Installation Practices" document.

This document covers such items as:-

- Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

- Safety (eye protection, rechecking torque procedure, etc.).

- Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

- Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Lexus dealer for a copy of this document.

##### Check Kit Content.

Check kit for content and damage.

One (1) Trunk Sill – Confirm correct vehicle model logo for Gas or Hybrid

One (1) Hardware Kit

##### Prepare the Vehicle.

Apply the parking brake.

Locate and remove the negative (-) battery terminal.

##### CAUTION: Do not touch the positive terminal with any tool during removal.

10mm Socket / Rachet or 10mm Wrench

Fig. 2-1

Negative

Battery Cable

Remove the negative (-) battery terminal using a 10mm wrench or socket
(Fig. 2-1).

Secure the negative (-) battery terminal so that it does not make intermittent contact with the battery terminals.

Fig. 3-1

Disassemble the Vehicle.

Protect the trunk opening while installing the Illuminated Trunk Sill (Fig 3-1).

Fig. 3-2

10mm Socket / Rachet

Remove two (2) 10mm bolts to remove D-Rings from the front of the trunk sill and retain for reassembly (Fig. 3-2 & Fig. 3-3).

Fig. 3-3

10mm Socket / Rachet

Fig. 3-4

Remove the trunk deck board (Fig. 3-4).

10mm Socket / Rachet

Fig. 3-5

Remove the clip located in the center of the trunk sill using a nylon panel removal tool and discard

Fig. 3-3

(Fig. 3-5).

Fig. 3-6

Move the foam tool block forward in the trunk (Fig. 3-6).

Fig. 3-7

Remove the trunk sill by pulling up on the trunk sill to disengage the six (6) clips and discard (Fig. 3-7).

Plastic pry tool

Fig. 3-8

Ensure all six (6) clips are removed from vehicle body. If clips remain on vehicle body use a nylon panel removal tool to remove clip(s) and discard clip(s) (Fig. 3-8).

Plastic pry tool

Fig. 3-9

Remove the clip attaching the LH quarter trim panel to the vehicle body near the LH trunk opening using a nylon panel removal tool and retain for reassembly (Fig. 3-9).

Plastic pry tool

Fig. 3-10

Remove the clip attaching the LH quarter trim panel to the vehicle body using a nylon panel removal tool and retain for reassembly (Fig. 3-10).

Fig. 3-11

Remove hook from LH quarter trim panel by pressing both buttons located on sides of hook and retain for reassembly (Fig. 3-11).

Fig. 3-12

10mm Socket / Rachet

Remove 10mm bolt to remove D-Ring from the LH quarter trim panel and retain for reassembly (Fig. 3-12).

Fig. 3-13

Plastic pry tool

Remove the clip attaching the LH quarter trim panel to the vehicle body using a nylon panel removal tool and retain for reassembly (Fig. 3-13).

Fig. 3-14

Plastic pry tool

Remove the LH storage compartment lid using a nylon panel removal tool to pry up near lid hinges and retain for reassembly (Fig. 3-14).

##### Install the Illuminated Trunk Sill.

Fig. 4-1

Place the illuminated trunk sill in the trunk (Fig. 4-1).

Fig. 4-2

Plastic pry tool

Remove trunk light by pressing on RH side of trunk light with a nylon panel removal tool to disengage the clip holding the light to the vehicle (Fig. 4-2).

Remove connector from trunk light and temporarily connect to illuminated trunk sill harness to confirm illumination.

Fig. 4-3

Install DCC fuse located in driver side of hood if not already installed (Fig 4-3).

Temporarily reconnect battery.

Fig. 4-4

Verify both logos and spot lights are illuminated (Fig. 4-4).

RH Logo for Gas Vehicle = ES350

RH Logo for Hybrid Vehicle = ES300h

Disconnect negative lead.

- CAUTION:  Do not touch the positive terminal.

Disconnect the trunk light harness from the illuminated trunk sill to continue with installation.

Fig. 4-5

Route the illuminated trunk sill harness behind the opening (Fig. 4-5).

Fig. 4-6

Route the illuminated trunk sill harness behind vehicle harness located near LH of trunk lid hinge (Fig. 4-6).

90mm x 60mm Foam Tape

Fig. 4-7

Locate fuse holder on the illuminated trunk sill harness and place 90mm x60mm foam tape over fuse holder prior to continuing with harness routing (Fig. 4-7).

Fig. 4-8

Trunk light opening

Opening No. 1

Continue routing the illuminated trunk sill harness through Opening #1 located near LH of trunk lid hinge until harness appears near Opening #2 (Fig. 4-8).

Opening No. 2

Opening No. 2

Opening No. 1

Trunk light opening

Continue routing the illuminated trunk sill harness through Opening #2 to the trunk light opening (Fig. 4-9).

Fig. 4-9

Fig. 4-10

Ensure the trunk light lock hook is straight up prior to installation. If the lock hook is not straight up, pull the hook to the right until lock hook is straight (Fig. 4-10).

Fig. 4-11

Connect the illuminated trunk sill harness male connector to the trunk light and attach vehicle side connector to the illuminated trunk sill harness (Fig. 4-11).

Fig. 4-12

Place 90mm x 30mm foam tape around the connectors (Fig. 4-12).

90mm x 30mm Foam Tape

Fig. 4-13

Install the trunk light (Fig. 4-13).

45mm x 30mm Foam Tape

Front of Vehicle

Fig. 4-14

Use two (2) 45mm x 30mm foam tapes to hold the illuminated trunk sill harness to the vehicle body (Fig. 4-14).

Fig. 4-15

Using two (2) zip ties attach the illuminated trunk sill harness to the vehicle harness running down near the LH trunk opening (Fig. 4-15).

3

4

Using two (2) zip ties attach the illuminated trunk sill harness to the vehicle harness running down near the LH trunk (Fig. 4-16).

Fig. 4-16

Using one (1) zip tie attach the illuminated trunk sill to the vehicle harness running across the trunk opening (Fig. 4-17).

5

Fig. 4-17

Fig. 4-18

6

Using one (1) zip tie attach the illuminated trunk sill to the vehicle harness running across the trunk opening (Fig. 4-18).

7

8

Fig. 4-19

9

Using three (3) zip ties attach the illuminated trunk sill harness to the vehicle harness running across the trunk opening
(Fig. 4-19).

11

Fig. 4-20

10

Using two (2) zip ties attach the illuminated trunk sill harness to the vehicle harness running across the trunk opening (Fig. 4-20).

##### Reassemble the Vehicle.

Reassemble the vehicle in the opposite order that it was disassembled.

Fig. 4-21

Remove DCC fuse located in driver side of hood if fuse was previously uninstalled (Fig 4-21).

Position the negative (-) battery cable at the original factory location.

Tighten the nut to 48 inlbf (5.4 Nm).

##### Torque: 48 inlbf (5.4 Nm)

- CAUTION:  Do not touch the positive terminal.

Accessory Function Checks

Illuminated Trunk Sill

Verify correct part was installed

Vehicle Function Checks

Trunk Light

Torque Audit

Verify logos and spot lights illuminate when trunk lid is open.

RH Logo for Gas Vehicle = ES350

RH Logo for Hybrid Vehicle = ES300h

Verify trunk light turns on when trunk lid is open.

Perform torque audit on battery

48 lbf-in (5.4 N-m)

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)