Part Number:	PT936-24150-XX

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Driver Door Guard (Reference Pink Sticker) |
| 2 | 1 | Passenger Door Guard (Reference Blue Sticker) |
| 3 |  |  |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | N/A |  |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | A/R | DIO: Simple Green All-Purpose Cleaner – Full Strength PPO: VDC Approved Cleaner |
| 2 | A/R | Lint-free towels |
| 3 | A/R | Blue painter’s masking tape |
| 4 | 1 | Scissors |

Conflicts

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| None |  |
| Special Tools | Notes |
| PPO Magnetic Template | IADS p/n = LRC15-D5-01 |
| Sealant Adjustment Tool | IADS p/n= HIG11-D5-03 |
| DIO Paper Door Templates | See last page of these Care must be taken when installing this accessory to ensure damage does not occur to the vehicle. The installation of this accessory should follow approved guidelines to ensure a quality installation. These guidelines can be found in the "Accessory Installation Practices" document. This document covers such items as:- • Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.). • Safety (eye protection, rechecking torque procedure, etc.). • Vehicle Disassembly/Reassembly (panel removal, part storage, etc.). • Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.). Vehicle Images shown below are generic and may not match actual vehicle. Please see your Lexus dealer for a copy of this document. instructions |
| Nylon Removal Tool(NRT) | SST p/n = 00002-06002-01 |
| Installation Tools | Notes |
| 12-oz. rubber mallet | IADS p/n =PRI10-D5-02 Vaughn, p/n = SF12 |
| Special Chemicals | Notes |
| Simple Green or VDC approved cleaner |  |

General Applicability

All

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 | Body Side Molding |
| 2 | Door Edge Guard |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

| FRONT DRIVER TEMPLATE | FRONT PASSENGER TEMPLATE | CLEANER | NYLON REMOVAL TOOL | MALLET |
| --- | --- | --- | --- | --- |
|  |  | TAPE | SCISSORS | LINT FREE |
| FRONT DOOR EDGE GUARDS |  |  |  |  |

ROTATE TOWEL AFTER EACH WIPE

ROTATE TOWEL AFTER EACH WIPE

DRY ALL DOOR EDGES

DRY ALL DOOR EDGES

CLEAN ALL DOOR EDGES

CLEAN ALL DOOR EDGES

1

1

2

x4

SQUEEK

ALIGN

ALIGN

4

4

3

3

5

5

START !!

START !!

ENSURE MALLET HEAD IS PERPENDICULAR TO THE EDGE OF THE DOOR

ENSURE MALLET HEAD IS PERPENDICULAR TO THE EDGE OF THE DOOR

6

6

~½”

~½”

7

7

~½”

~½”

8

8

USE YELLOW SIDE OF MALLET HEAD

~1”

THUD

=

THUD

6

&

7

7

SEE PAGE 8 FOR INSTALLATION CHECKLIST

REPEAT ALL DOORS

9

DEG REMOVAL

DEG REMOVAL

START !!

START !!

~4”

~4”

~½”

~½”

~½”

~½”

THIS PAGE PPO ONLY

THIS PAGE PPO ONLY

Fig. 3-1

Sealant Adjustment Tool HIG11-D5-01

Fig. 3-1

Sealant Adjustment Tool HIG11-D5-01

NOTE:  The adjustment tool can be used if sealant is close to the door’s edge.  This must be done to allow for proper Door Edge Guard installation (Fig. 3-1).

Fig. 3-2

Depth gage (Side view)

Fig. 3-2

Depth gage (Side view)

Align the adjustment tool to the edge of the door, with the angle edge positioned toward the inside of the door (Fig. 3-2).

Use the adjustment tool as a depth gage down the entire length of the door’s edge. This is to verify that the sealant material is back away from the edge of the door to allow for proper Door Edge Guard installation.

Fig. 3-3

Angle tip in contact with sealant

Fig. 3-3

Angle tip in contact with sealant

Align the adjustment tool to the edge of the door, with the angle edge positioned toward the inside of the door (Fig. 3-3).

With the adjustment tool angle tip in contact with the sealant material, push the adjustment tool inward so that the notched section of the tool is in complete contact with the door’s edge.  Continue down the door’s edge or sections of the door’s edge that requires adjustment.

Accessory Function Checks

Confirm all Door Edge Guards are fully seated.

Vehicle Function Checks

Vehicle door operation.

Look for a fairly consistent gap between the DEG and adjoining vehicle panels.

NOTE:  Normal vehicle manufacturing tolerances may exist that are not a consequence of an improperly seated DEG installation

Confirm both doors open and close without interference from DEG or Body Side Molding (if installed).

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.

0”

1”

2”

3”

4”

5”

0”

1”

2”

3”

4”

5”

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.

0”

1”

2”

3”

4”

5”

0”

1”

2”

3”

4”

5”

0”

1”

2”

3”

4”

5”

0”

1”

2”

3”

4”

5”

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.

Dealer Installation Note: Check the above scale to ensure it is 1:1.   Adjust on photocopier if it is not to scale. If this is a pdf document, set the “Page Scaling” to “None” in the Print dialogue box prior to printing.